SMART PARKING
DETECTION SYSTEM

Full project report, visual results, formulas, and code walkthrough

Prepared 12 October 2026  |  Inference batch: 20261012_223506

CRITICAL EVALUATION NOTE

The high reported test metrics measure parking-slot detection, not correct/incorrect parking decisions. The dataset has no manually labeled violation class. Red overlays are possible-violation alerts from geometry and require human verification.

Includes all eight latest input/output image pairs and teacher-ready technical explanations.

# 1. Executive summary

This project is a computer-vision prototype for parking-lot monitoring. A YOLOv8 model predicts parking-space boxes and labels them empty or occupied. A separate pretrained model predicts vehicle boxes. A geometry-based rule then compares vehicles with parking spaces to identify candidate free spaces and possible parking violations.

Latest batch: 8 attached images processed with slot_det_best.pt and yolov8n.pt on CPU.

Combined output: 116 vehicle detections, 817 slot predictions, 581 free-slot predictions, 17 red possible-violation predictions, 31 vehicles labeled confidently correct (and omitted from red/green-only render), and 68 uncertain detections.

These are prediction counts, not accuracy measurements. The uploaded scenes do not include ground-truth vehicle or violation annotations.

The slot detector has strong reported test metrics on the parking-space task. Wrong-parking precision, recall, F1, and accuracy remain unavailable without labeled violations.

All 8 input images and all 8 corresponding annotated outputs are included later in this report. Images 5 and 6 are selected as visually plausible red-alert examples, not certified correct classifications.

# 2. Project problem, aims, and scope

## Problem

A parking scene has many adjacent spaces and vehicles, often viewed obliquely from a high camera. Determining open spaces and spotting a vehicle that crosses bay boundaries is difficult to automate because cars may be small, slot markings may be faint, and the perspective changes across the image.

## Aims

Detect parking spaces and classify the dataset labels space-empty / space-occupied.

Detect visible vehicles using a general-purpose vehicle model.

Estimate vehicle-to-space overlap and flag likely multi-space intrusions.

Keep ambiguous cases uncertain instead of forcing a correct/wrong decision.

Render visual overlays that a human operator can inspect.

## Scope / exclusions

This is a still-image prototype, not a deployed enforcement system. It does not identify drivers, read license plates, determine local parking law, or prove a violation. It does not use temporal tracking in the attached-image batch.

# 3. Dataset, labels, and what they mean

The project uses a PKLot-derived Roboflow export documented as 12,416 640x640 images: 8,691 train, 2,483 validation, and 1,242 test. The two trained classes are parking-space categories:

| Class id | Class name | Ground-truth meaning |
| --- | --- | --- |
| 0 | space-empty | An annotated parking-space region labeled empty. |
| 1 | space-occupied | An annotated parking-space region labeled occupied. |

The labels are for parking spaces, not separate car silhouettes.

There is no wrong-parking/violation class in this export.

Occupied space does not mean “incorrectly parked”; it means only that the space annotation is occupied.

The annotations are axis-aligned boxes; real parking bays may be rotated or perspective-distorted.

# 4. Model architecture and inference flow

| Stage | Model / method | What it produces |
| --- | --- | --- |
| 1. Parking-space detection | YOLOv8s checkpoint: weights/slot_det_best.pt | space-empty and space-occupied bounding boxes. |
| 2. Vehicle detection | COCO-pretrained YOLOv8n: weights/yolov8n.pt | Candidate vehicle bounding boxes; not custom-trained on these submitted camera images. |
| 3. Geometry | Shapely polygon/rectangle intersections | Inside, outside, slot-coverage, and adjacent-slot intrusion ratios. |
| 4. Rule classification | Threshold-based classifier | WRONG_PARKING, CORRECT_PARKING, or UNCERTAIN. |
| 5. Output rendering | OpenCV rectangles and labels | Green = predicted usable free slot; red = possible incorrect parking; correct/uncertain cars omitted. |

## End-to-end sequence

Read image and run the slot detector.

Read image and run the vehicle detector.

For each vehicle, calculate overlap with every predicted slot and choose the largest eligible vehicle-inside ratio as its primary slot.

Check whether the car appears much larger than a slot and intrudes into a second slot beyond a tolerance band.

If no straddle is found, check whether a sufficiently large share of the car box lies outside the assigned space.

Emit a red flag only when a configured rule passes; otherwise classify as correct only for a strong single-space fit, or leave uncertain.

Draw green boxes for free-space predictions and red boxes for possible violations.

# 5. Geometry formulas used by the classifier

Let B be the detected vehicle bounding box, S a parking-slot bounding box/polygon, A(X) the area of region X, and ∩ the geometric intersection.

| Name | Formula | Plain-English meaning |
| --- | --- | --- |
| Vehicle inside ratio, R_vehicle | A(B ∩ S) / A(B) | Fraction of the detected vehicle rectangle lying inside one slot. |
| Slot coverage ratio, R_slot | A(B ∩ S) / A(S) | Fraction of the slot rectangle covered by the vehicle box. |
| Outside ratio, R_outside | 1 - R_vehicle | Estimated fraction of vehicle box outside that one slot. |
| Intersection over Union, IoU | A(B ∩ S) / A(B ∪ S) | Overlap relative to combined vehicle + slot box area. |
| Second-slot intrusion, R_intrude | A(B ∩ (S2 minus buffered S1)) / A(B) | Vehicle fraction inside adjacent slot S2 after excluding primary slot S1 plus tolerance buffer. |

## Decision rules in the current configuration

Primary slot assignment: R_vehicle >= 0.30 (slot_assign_min).

Single-slot red rule: confident vehicle and R_outside >= 0.70 (wrong_parking_min_outside_ratio).

Straddle red rule: car is at least 1.50x the assigned slot size in one dimension and R_intrude >= 0.20; a separate stricter fallback may apply for strongly elongated boxes.

Correct/blue internal status (not drawn in the latest outputs): strong fit thresholds or at least 0.50 inside one unambiguous slot.

No strong assignment, low confidence, competing slots, or borderline overlap -> UNCERTAIN.

Important caveat: These are ratios of model-predicted rectangles, not true vehicle footprints or legal boundaries. A geometric rule can be wrong when either detector box is inaccurate.

# 6. Important code excerpts for explaining the project

The snippets below are short, teacher-readable excerpts matching the project logic. They illustrate the equations and rules without requiring a presentation audience to read the entire source repository.

## 6.1 Compute vehicle/slot overlap ratios

# For vehicle region B and slot region S: R_vehicle = area(intersection(B, S)) / area(B) R_slot = area(intersection(B, S)) / area(S) R_outside = 1.0 - R_vehicle # Source implementation concept (src/geometry.py): intersection = vehicle_polygon.intersection(slot_polygon).area r_vehicle = intersection / vehicle_polygon.area r_slot = intersection / slot_polygon.area r_outside = 1.0 - r_vehicle

In the code, degenerate zero-area boxes are guarded so the division is not performed when the relevant area is zero.

## 6.2 Choose primary slot

metrics = [compute_overlap_metrics(vehicle, slot) for slot in slots] metrics.sort(key=lambda item: item.r_vehicle, reverse=True) primary = metrics[0] if metrics and metrics[0].r_vehicle >= slot_assign_min else None

The slot that contains the largest fraction of the vehicle box is the primary candidate, provided it passes the minimum assignment threshold.

## 6.3 Adjacent-slot intrusion and red classification

tolerance = min(vehicle_width, vehicle_height) * boundary_tolerance_ratio intrusion = vehicle_intrusion_ratio_beyond_tolerance( vehicle, adjacent_slot, primary_slot, tolerance ) if car_is_large_relative_to_slot and intrusion >= 0.20: status = "WRONG_PARKING" # possible straddle alert elif confident and primary.r_outside >= 0.70: status = "WRONG_PARKING" # most of vehicle box outside one slot elif strong_single_slot_fit and not_competing_slot: status = "CORRECT_PARKING" else: status = "UNCERTAIN"

Simplification note: The production implementation also checks vehicle confidence, slot-size ratio, competing slot overlap, and elongated-box fallback. The excerpt is explanatory pseudocode, not a complete drop-in implementation.

## 6.4 Main application call flow

slot_detector, vehicle_detector = load_models( slot_weights="weights/slot_det_best.pt", vehicle_weights="weights/yolov8n.pt", device="cpu", ... ) frame_result, annotated, latency_ms = process_image( image, slot_detector, vehicle_detector, thresholds ) # frame_result contains vehicle and slot classifications # annotated image is drawn and saved as a PNG

# 7. Training and reported evaluation metrics

## Training setup

| Parameter | Recorded configuration |
| --- | --- |
| Model / task | YOLOv8s / object detection |
| Epochs | 50 |
| Image size | 640 |
| Batch size | 16 |
| Seed | 42 |
| Classes | space-empty, space-occupied |
| Checkpoint | weights/slot_det_best.pt |

## Held-out slot-test metrics (Kaggle run)

| Metric | Value | Meaning |
| --- | --- | --- |
| Precision | 99.84% | Among predicted slot boxes, share matching a labeled slot under evaluation criteria. |
| Recall | 97.97% | Among labeled slots, share found by the detector. |
| mAP@0.50 | 97.95% | Mean average precision at IoU 0.50 for slot classes. |
| mAP@0.50:0.95 | 97.70% | Average mAP across IoU thresholds 0.50 through 0.95, as recorded in the Kaggle metrics file. |

Source: new_model/slot_metrics.json. These numbers apply only to the slot detector on its recorded test split; they are not vehicle or violation metrics.

## Separate local 100-image evaluation (do not merge with above)

| Class / metric | Precision | Recall | mAP@0.50 | mAP@0.50:0.95 |
| --- | --- | --- | --- | --- |
| space-empty | 93.1% | 97.3% | 98.2% | 66.0% |
| space-occupied | 96.7% | 94.0% | 98.7% | 59.9% |
| All slot classes | 94.9% | 95.6% | 98.4% | 62.9% |

Source: outputs/evaluation/eval_log.txt and outputs/evaluation/slot_metrics.json. This is a separate local evaluation of 100 images using a different split/metric record. Keep it distinct from the 1,242-image Kaggle test result.

## Metrics that cannot currently be claimed

Wrong-parking precision: unavailable.

Wrong-parking recall: unavailable.

Wrong-parking F1 / accuracy / confusion matrix: unavailable.

Custom camera-scene vehicle detector precision/recall: not established by the supplied parking-slot labels.

To compute violation metrics, each test vehicle must have a verified ground-truth label (for example, valid / violation / uncertain) and be matched consistently to model predictions.

# 8. Results from latest eight attached images

Batch ID 20261012_223506. All images were processed using the current slot and vehicle weights, CPU inference, thresholds in configs/thresholds.yaml, vehicle image size 1280, slot image size 640, and tiled_inference=false.

| # | Size | Veh. | Slots | Free* | Red* | Correct* | Uncertain | Latency |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 768x768 | 16 | 100 | 76 | 3 | 2 | 11 | 2.36s |
| 2 | 768x768 | 20 | 103 | 72 | 3 | 4 | 13 | 2.05s |
| 3 | 768x768 | 6 | 104 | 90 | 2 | 1 | 3 | 1.20s |
| 4 | 768x768 | 8 | 104 | 88 | 2 | 1 | 5 | 1.37s |
| 5 | 768x768 | 2 | 106 | 99 | 1 | 0 | 1 | 0.90s |
| 6 | 768x768 | 2 | 100 | 89 | 1 | 0 | 1 | 0.86s |
| 7 | 768x768 | 54 | 100 | 38 | 5 | 19 | 30 | 4.30s |
| 8 | 640x640 | 8 | 100 | 29 | 0 | 4 | 4 | 1.26s |
| TOTAL | 8 imgs | 116 | 817 | 581 | 17 | 31 | 68 | 14.29s |

*Every quantity marked with an asterisk in this table is a model prediction/output count, not a verified truth count. “Correct omitted” means the classifier assigned CORRECT_PARKING and the vehicle box was not drawn in the red/green-only overlay.

## Aggregate interpretation

The system emitted 17 red possible-violation alerts across 116 detected vehicles.

68 vehicle detections stayed uncertain; they must not be counted as correct or as wrong.

581 predicted free slots were drawn across the eight images; without annotated slot truth for these images, free-slot precision/recall on this batch cannot be calculated.

A count of zero red predictions on an image would not prove there are no violations; the classifier can miss violations or leave vehicles uncertain.

# 9. Representative images and all eight input/output pairs

The next eight pages contain every original input next to its corresponding model output. The overlays show green predicted-free slots and red possible incorrect-parking flags. In this dataset, no car-level ground-truth violation labels exist, so examples that appear visually plausible are not guaranteed correct classifications.

# Image pair 1 - Latest inference example

16 vehicle detections | 100 slot predictions | 76 free-slot predictions | 3 red alerts | 11 uncertain

Qualitative model output only; not a labeled validation example.

| INPUT IMAGE Pasted Image.png | MODEL OUTPUT latest_20261012_223506_01_Pasted_Image_1.png |
| --- | --- |

Teacher explanation: The left panel is the original scene. The right panel overlays predicted empty-space boxes in green and possible violation boxes in red. Uncertain vehicle boxes are intentionally not drawn in this simplified output.

# Image pair 2 - Latest inference example

20 vehicle detections | 103 slot predictions | 72 free-slot predictions | 3 red alerts | 13 uncertain

Qualitative model output only; not a labeled validation example.

| INPUT IMAGE Pasted Image 2.png | MODEL OUTPUT latest_20261012_223506_02_Pasted_Image_2.png |
| --- | --- |

Teacher explanation: The left panel is the original scene. The right panel overlays predicted empty-space boxes in green and possible violation boxes in red. Uncertain vehicle boxes are intentionally not drawn in this simplified output.

# Image pair 3 - Latest inference example

6 vehicle detections | 104 slot predictions | 90 free-slot predictions | 2 red alerts | 3 uncertain

Qualitative model output only; not a labeled validation example.

| INPUT IMAGE Pasted Image 3.png | MODEL OUTPUT latest_20261012_223506_03_Pasted_Image_3.png |
| --- | --- |

Teacher explanation: The left panel is the original scene. The right panel overlays predicted empty-space boxes in green and possible violation boxes in red. Uncertain vehicle boxes are intentionally not drawn in this simplified output.

# Image pair 4 - Latest inference example

8 vehicle detections | 104 slot predictions | 88 free-slot predictions | 2 red alerts | 5 uncertain

Qualitative model output only; not a labeled validation example.

| INPUT IMAGE Pasted Image 4.png | MODEL OUTPUT latest_20261012_223506_04_Pasted_Image_4.png |
| --- | --- |

Teacher explanation: The left panel is the original scene. The right panel overlays predicted empty-space boxes in green and possible violation boxes in red. Uncertain vehicle boxes are intentionally not drawn in this simplified output.

# Image pair 5 - Latest inference example

2 vehicle detections | 106 slot predictions | 99 free-slot predictions | 1 red alerts | 1 uncertain

Selected as a visually plausible example of the geometry-based alert on a vehicle near bay boundaries. Do not label “verified correct” without human annotation against the exact parking rules.

| INPUT IMAGE Pasted Image 5.png | MODEL OUTPUT latest_20261012_223506_05_Pasted_Image_5.png |
| --- | --- |

Teacher explanation: The left panel is the original scene. The right panel overlays predicted empty-space boxes in green and possible violation boxes in red. Uncertain vehicle boxes are intentionally not drawn in this simplified output.

# Image pair 6 - Latest inference example

2 vehicle detections | 100 slot predictions | 89 free-slot predictions | 1 red alerts | 1 uncertain

Selected as a visually plausible example of the geometry-based alert on a vehicle near bay boundaries. Do not label “verified correct” without human annotation against the exact parking rules.

| INPUT IMAGE Pasted Image 6.png | MODEL OUTPUT latest_20261012_223506_06_Pasted_Image_6.png |
| --- | --- |

Teacher explanation: The left panel is the original scene. The right panel overlays predicted empty-space boxes in green and possible violation boxes in red. Uncertain vehicle boxes are intentionally not drawn in this simplified output.

# Image pair 7 - Latest inference example

54 vehicle detections | 100 slot predictions | 38 free-slot predictions | 5 red alerts | 30 uncertain

Dense-scene stress example. Many detections remain uncertain; red overlays should be manually reviewed.

| INPUT IMAGE Pasted Image 7.png | MODEL OUTPUT latest_20261012_223506_07_Pasted_Image_7.png |
| --- | --- |

Teacher explanation: The left panel is the original scene. The right panel overlays predicted empty-space boxes in green and possible violation boxes in red. Uncertain vehicle boxes are intentionally not drawn in this simplified output.

# Image pair 8 - Latest inference example

8 vehicle detections | 100 slot predictions | 29 free-slot predictions | 0 red alerts | 4 uncertain

Qualitative model output only; not a labeled validation example.

| INPUT IMAGE Pasted Image 8.jpe | MODEL OUTPUT latest_20261012_223506_08_Pasted_Image_8.png |
| --- | --- |

Teacher explanation: The left panel is the original scene. The right panel overlays predicted empty-space boxes in green and possible violation boxes in red. Uncertain vehicle boxes are intentionally not drawn in this simplified output.

# 10. Strengths, limits, and risks

| Strength | Limitation / risk |
| --- | --- |
| Trained space detector distinguishes empty and occupied slots. | High slot metrics do not transfer to vehicle/violation accuracy. |
| Geometry provides an explainable rule for a red alert. | A box is not a true car footprint; perspective and slot-box error can cause false reds or misses. |
| Uncertain state prevents some forced classifications. | The 68 uncertain detections in the latest batch reduce coverage; omitted from red-only visualization. |
| Image output is easy to inspect. | Visual examples without truth labels cannot prove precision, recall, or accuracy. |
| Thresholds are configurable. | Changing thresholds trades missed alerts against false alarms; requires camera-specific calibration. |

## Recommended development plan

Collect and annotate representative target-camera scenes with vehicle boxes, exact slot polygons, and valid/violation/uncertain labels.

Create train/validation/test splits by scene, camera, date, or lot to prevent near-duplicate leakage.

Fine-tune a vehicle detector using true vehicle-instance labels from the target camera, especially small aerial vehicles.

Use calibrated perspective-aware slot polygons rather than one-size axis-aligned rectangles.

Tune thresholds only on labeled validation data.

On the held-out set report per-class precision, recall, F1, AP, confusion matrix, false alarms per image, and missed violations.

Have a human verify alerts before any enforcement use.

# 11. Teacher-ready presentation outline and short script

## Slide 1 - Project title

Slide content: Smart Parking Detection System

Speaker script: “This is a prototype that combines parking-space detection, vehicle detection, and geometric rules to flag possible parking violations.”

## Slide 2 - Problem

Slide content: Parking-lot occupancy and boundary violations are difficult to review manually.

Speaker script: “The system aims to identify candidate free spaces and possible bay intrusions from a camera image.”

## Slide 3 - Data

Slide content: 12,416 images; 8,691 train / 2,483 validation / 1,242 test; space-empty and space-occupied labels.

Speaker script: “The training data labels spaces, not individual vehicles or violations.”

## Slide 4 - Architecture

Slide content: Slot detector -> vehicle detector -> overlap geometry -> decision -> overlay.

Speaker script: “The first model predicts spaces, a pretrained detector predicts vehicles, and rules compare their boxes.”

## Slide 5 - Formula

Slide content: R_vehicle = intersection area / vehicle area; R_outside = 1 - R_vehicle.

Speaker script: “If a confident car lies mostly outside its assigned slot, or significantly intrudes into an adjacent bay, it may receive a red candidate alert.”

## Slide 6 - Slot-model results

Slide content: Kaggle full test: P 99.84%, R 97.97%, mAP50 97.95%, mAP50-95 97.70%.

Speaker script: “These are space-detection metrics only, not violation-classification results.”

## Slide 7 - Qualitative images

Slide content: Show a selected input/output pair, preferably image pair 5 or 6, with caveat.

Speaker script: “This example shows the model output; red boxes need human verification because the dataset has no violation labels.”

## Slide 8 - Batch summary

Slide content: 8 images: 116 vehicle detections; 17 red flags; 68 uncertain.

Speaker script: “These are raw prediction counts; they cannot be interpreted as precision or recall.”

## Slide 9 - Limitations

Slide content: No labeled violation test set; perspective/box mismatch; vehicle detector not specialized for these aerial images.

Speaker script: “The next step is to collect annotated target-camera data and evaluate false positives and missed violations.”

## Slide 10 - Conclusion

Slide content: Working prototype, strong slot benchmark, violation classifier still requires validation.

Speaker script: “The system can help a person review parking scenes, but it is not yet an independently validated enforcement tool.”

# 12. Teacher questions and concise answers

| Question | Short answer |
| --- | --- |
| What does the model detect? | The trained custom model detects empty and occupied parking spaces. A pretrained COCO model detects vehicles. |
| How does it call something wrongly parked? | It compares the vehicle box with predicted slot boxes using inside/outside overlap and adjacent-slot intrusion thresholds. |
| What does 99.84% precision mean? | For the recorded full test run, it is slot-box precision under the detector’s matching criteria. It is not the precision of red violation alerts. |
| How many wrong-parking labels were used to train? | None in the documented PKLot export. Wrong parking is a hand-coded geometry heuristic, not a learned class. |
| Can we report wrong-parking accuracy now? | No. We need manually labeled violations and a held-out evaluation before computing precision, recall, F1, or accuracy. |
| Why are some cars uncertain or not detected? | The vehicle model is general-purpose; small objects, occlusion, perspective, weak slot detections, and ambiguous overlaps can reduce confidence. |
| Is the output ready for automated enforcement? | No. Treat red boxes as review candidates; human verification and validated camera-specific metrics are required. |

# 13. Reproducibility and artifact paths

| Artifact | Project path |
| --- | --- |
| Full 8-image summary | myfold/attached_images/latest_20261012_223506_summary.json |
| Eight annotated outputs | myfold/attached_images/latest_20261012_223506_01_Pasted_Image_1.png through ..._08_Pasted_Image_8.png |
| Slot model metrics | new_model/slot_metrics.json |
| Separate local class metrics | outputs/evaluation/eval_log.txt and outputs/evaluation/slot_metrics.json |
| Thresholds | configs/thresholds.yaml |
| Classifier implementation | src/parking_classifier.py |
| Geometry implementation | src/geometry.py |
| Inference implementation | src/inference.py |
| Training configuration | new_model/args.yaml |

Inference run configuration: local Python environment, CPU, current weights, thresholds loaded from configs/thresholds.yaml, vehicle image size 1280, slot image size 640, tiled inference disabled. The run summary records per-image counts and latency.

# 14. Final conclusion

The system demonstrates an end-to-end prototype and the trained parking-space detector has strong recorded benchmark metrics. The attached-image run also demonstrates explainable geometry-based candidate alerts. However, the project has not established wrong-parking precision or recall because the available annotations contain no verified violations. The scientifically accurate conclusion is that this system flags possible violations for human review; a labeled camera-specific evaluation is required before making accuracy claims or using it for enforcement.